Net 1060   Introduction to Networks     	 Lab:  #   2.9.2

Name:

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. Ensure you paste the screen shot of your score page at the bottom of the document. Even if this does not let you see your grade, still take a screen shot of the score page showing congratulations “your name” you have completed the exercise is pasted at the bottom of this document. You will then need to upload both this word document and your packet tracer file to the assignments link within Netacad. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

First, I set up the switches and PCs physically with cables. Then, I configured the IP addresses for both the switches and the PCs. Then, I configured the basic settings for the switches. Finally, I tested the connections by sending pings to both the switches and the PCs.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned how to put everything together and get a small network up and running. I just did a little bit of everything that I previously learned.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you    learned from that assistance.

# Packet Tracer - Basic Switch and End Device Configuration - Physical Mode

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 1 | 192.168.1.1 | 255.255.255.0 |
| S2 | VLAN 1 | 192.168.1.2 | 255.255.255.0 |
| PC-A | NIC | 192.168.1.10 | 255.255.255.0 |
| PC-B | NIC | 192.168.1.11 | 255.255.255.0 |

Blank Line - no additional information

# Objectives

Part 1: Set Up the Network Topology

Part 2: Configure PC Hosts

Part 3: Configure and Verify Basic Switch Settings

# Background / Scenario

In this Packet Tracer Physical Mode (PTPM) activity, you will build a simple network with two hosts and two switches. You will also configure basic settings including hostname, local passwords, and login banner. Use show commands to display the running configuration, IOS version, and interface status. Use the copy command to save device configurations.

You will apply IP addressing for to the PCs and switches to enable communication between the devices. Use the ping utility to verify connectivity.

# Instructions

## Set Up the Network Topology

Power on the PCs and cable the devices according to the topology. To select the correct port on a switch, right click and select Inspect Front. Use the Zoom tool, if necessary. Float your mouse over the ports to see the port numbers. Packet Tracer will score the correct cable and port connections.

There are several switches, routers, and other devices on the Shelf. Click and drag switches S1 and S2 to the Rack. Click and drag two PCs to the Table.

Power on the PCs.

On the Cable Pegboard, click a Copper Cross-Over cable. Click the FastEthernet0/1 port on S1 and then click the FastEthernet0/1 port on S2 to connect them. You should see the cable connecting the two ports.

On the Cable Pegboard, click a Copper Straight-Through cable. Click the FastEthernet0/6 port on S1 and then click the FastEthernet0 port on PC-A to connect them.

On the Cable Pegboard, click a Copper Straight-Through cable. Click the FastEthernet0/18 port on S2 and then click the FastEthernet0 port on PC-B to connect them.

Visually inspect network connections. Initially, when you connect devices to a switch port, the link lights will be amber. After a minute or so, the link lights will turn green.

## Configure PC Hosts

Configure static IP address information on the PCs according to the Addressing Table.

Click PC-A > Desktop > IP Configuration. Enter the IP address for PC-A (192.168.1.10) and the subnet mask (255.255.255.0), as listed in the IP addressing table. You can leave default gateway blank at this time because there is no router attached to the network.

Close the PC-A window.

Repeat the previous steps to assign the IP address information for PC-B, as listed in the Addressing Table.

Click PC-A > Desktop > Command Prompt. Use the ipconfig /all command at the prompt to verify settings.

Enter ping 192.168.1.11 at the prompt to test the connectivity to PC-B. The ping should be successful, as shown in the following output. If the ping is not successful, check the configurations on both of the PCs and troubleshoot as necessary.

Packet Tracer PC Command Line 1.0

C:\> ping 192.168.1.11

Pinging 192.168.1.11 with 32 bytes of data:

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.11:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\>

## Configure and Verify Basic Switch Settings

On the Cable Pegboard, click a Console cable. Connect the console cable between S1 and PC-A.

Establish a console connection to the switch S1 from PC-A using the Packet Tracer generic Terminal program (PC-A > Desktop > Terminal). Press ENTER to get the Switch> prompt.

You can access all switch commands in privileged EXEC mode. The privileged EXEC command set includes those commands contained in user EXEC mode, as well as the configure command through which access to the remaining command modes are gained. Enter privileged EXEC mode by entering the enable command.

Open Configuration Window

The prompt changed from Switch> to Switch# which indicates privileged EXEC mode. Enter global configuration mode.

The prompt changed to Switch(config)# to reflect global configuration mode. Give the switch a name according to the Addressing Table.

Enter local passwords. Use class as the privileged EXEC password and cisco as the password for console access.

Configure and enable the VLAN 1 interface according to the Addressing Table.

A login banner, known as the message of the day (MOTD) banner, should be configured to warn anyone accessing the switch that unauthorized access will not be tolerated. Configure an appropriate MOTD banner to warn about unauthorized access.

Save the configuration to the startup file on non-volatile random access memory (NVRAM).

Display the current configuration.

Display the IOS version and other useful switch information.

Display the status of the connected interfaces on the switch.

Close Configuration Window.

Repeat the previous steps to configure switch S2. Make sure the hostname is configured as S2.

Record the interface status for the following interfaces.

| Interface | S1 Status | S1 Protocol | S2 Status | S2 Protocol |
| --- | --- | --- | --- | --- |
| F0/1 | Up | Up | Up | Up |
| F0/6 | Up | Up | Up | Up |
| F0/18 | Up | Up | Up | Up |
| VLAN 1 | up | Up | Up | Up |

Blank Line - no additional information

From a PC, ping S1 and S2. The pings should be successful.

From a switch, ping PC-A and PC-B. The pings should be successful.

# Reflection Question

Why are some FastEthernet ports on the switches up while others are down?

Most of the FastEthernet ports aren’t connected to anything. The only ones that are up are the connections I put in.

What could prevent a ping from being sent between the PCs?

A cable being disconnected from the switch or the wrong IP address being entered.here.

End of Document