Net 1061 Switching, Routing, and Wireless Essentials

Name:

Lab:  #          1.6.2 Configure Basic Router Settings

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. You will also have to copy and paste your running configurations of each device configured to the bottom of this document. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

In this lab, I wired the router, switch, and PCs. Then I configured the ip addresses. Then, I did the basic configurations on the router, as well as SSH. Then, I verified connectivity. Finally, I connected to the router using SSH and got the router information.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned how to put all the skills I know and apply them in a real setting. I learned how to connect using SSH. I also learned how to configure two different PCs using a single PC.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t save my configurations from the first day so I had to restart.

2. Using complete sentences describe how you solved your problem(s).

I redid the lab.

3. Using complete sentences explain if you needed any assistance with the lab; then list what you learned from that assistance.

# Lab - Configure Basic Router Settings

# Topology

# Addressing Table

| Device | Interface | IP Address / Prefix | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.0.1 /24 | N/A |
| R1 | G0/0/0 | 2001:db8:acad::1 /64 | N/A |
| R1 | G0/0/0 | fe80::1 | N/A |
| R1 | G0/0/1 | 192.168.1.1 /24 | N/A |
| R1 | G0/0/1 | 2001:db8:acad:1::1 /64 | N/A |
| R1 | G0/0/1 | fe80::1 | N/A |
| R1 | Loopback0 | 10.0.0.1 /24 | N/A |
| R1 | Loopback0 | 2001:db8:acad:2::1 /64 | N/A |
| R1 | Loopback0 | fe80::1 | N/A |
| PC-A | NIC | 192.168.1.10 /24 | 192.168.1.1 |
| PC-A | NIC | 2001:db8:acad:1::10 /64 | fe80::1 |
| PC-B | NIC | 192.168.0.10 /24 | 192.168.0.1 |
| PC-B | NIC | 2001:db8:acad::10 /64 | fe80::1 |

# Objectives

Part 1: Set Up the Topology and Initialize Devices

Cable equipment to match the network topology.

Initialize and restart the router and switch.

Part 2: Configure Devices and Verify Connectivity

Assign static IPv4 and IPv6 information to the PC interfaces.

Configure basic router settings.

Configure the router for SSH.

Verify network connectivity.

Part 3: Display Router Information

Retrieve hardware and software information from the router.

Interpret the output from the startup configuration.

Interpret the output from the routing table.

Verify the status of the interfaces.

# Background / Scenario

This is a comprehensive lab to review previously covered IOS router commands. In Parts 1 and 2, you will cable the equipment and complete basic configurations and interface settings on the router.

In Part 3, you will use SSH to connect to the router remotely and utilize the IOS commands to retrieve information from the device to answer questions about the router.

For review purposes, this lab provides the commands necessary for specific router configurations.

Note: The routers used with CCNA hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). The switches used in the labs are Cisco Catalyst 2960s with Cisco IOS Release 15.2(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Make sure that the router and switch have been erased and have no startup configurations. Consult with your instructor for the procedure to initialize and reload a router and switch.

# Required Resources

1 Router (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable)

2 PCs (Windows with a terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Note: The Gigabit Ethernet interfaces on Cisco 4221 routers are autosensing and an Ethernet straight-through cable may be used between the router and PC-B. If using another model Cisco router, it may be necessary to use an Ethernet crossover cable.

# Instructions

## Set Up the Topology and Initialize Devices

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

Power on all the devices in the topology.

### Initialize and reload the router and switch.

## Configure Devices and Verify Connectivity

### Configure the PC interfaces.

Configure the IP address, subnet mask, and default gateway settings on PC-A.

Configure the IP address, subnet mask, and default gateway settings on PC-B.

### Configure the router.

Open configuration window

Console into the router and enable privileged EXEC mode.

Enter configuration mode.

Assign a device name to the router.

Set the router’s domain name as ccna-lab.com.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Encrypt the plaintext passwords.

Configure the system to require a minimum 12-character password.

Configure the username SSHadmin with an encrypted password of 55Hadm!n2020.

Generate a set of crypto keys with a 1024 bit modulus

Assign the privileged EXEC password to $cisco!PRIV*

Assign $cisco!!CON* as the console password, configure sessions to disconnect after four minutes of inactivity, and enable login.

Assign $cisco!!VTY* as the vty password, configure the vty lines to accept SSH connections only, configure sessions to disconnect after four minutes of inactivity, and enable login using the local database.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Enable IPv6 Routing

Configure all three interfaces on the router with the IPv4 and IPv6 addressing information from the addressing table above. Configure all three interfaces with descriptions. Activate all three interfaces.

The router should not allow vty logins for two minutes if three failed login attempts occur within 60 seconds.

Set the clock on the router.

Save the running configuration to the startup configuration file.

#### Question:

What would be the result of reloading the router prior to completing the copy running-config startup-config command?

Type your answers here.

Close configuration window

### Verify network connectivity.

Using the command line at PC-A, ping the IPv4 and IPv6 addresses for PC-B.

Note: It may be necessary to disable the PCs firewall.

#### Question:

Were the pings successful?

Yes, the pings were successful.  your answers here.

Remotely access R1 from PC-A using the Tera Term SSH client.

Using Tera Term on PC-A, open an SSH session to the R1 Loopback interface IPv4 address. Ensure that the SSH radio button is selected and then click OK to connect to the router. Log in as SSHadmin with the password 55Hadm!n2020.

#### Question:

- Was remote access successful?

- Yes, the connection was successful.

Type your answers here.

Using Tera Term on PC-A, open an SSH session to the R1 Loopback interface IPv6 address. Ensure that the SSH radio button is selected and then click OK to connect to the router. Log in as SSHadmin with the password 55Hadm!n2020. Note: The IPv6 address should be surrounded with square brackets, i.e. [IPv6 address]

#### Questions:

- Was remote access successful?

Yes, it was successful.Type your answers here.

Why is the Telnet protocol considered to be a security risk?

It does not offer a secure connection like SSH. your answers here.

## Display Router Information

In Part 3, you will use show commands from an SSH session to retrieve information from the router.

### Establish an SSH session to R1.

Using Tera Term on PC-B, open an SSH session to the R1 Loopback interface IPv6 address and log in as SSHadmin with the password 55Hadm!n2020.

### Retrieve important hardware and software information.

Use the show version command to answer questions about the router.

#### Questions:

What is the name of the IOS image that the router is running?

Cisco IOS XE Software, Version 16.06.04 is the image.  your answers here.

How much non-volatile random-access memory (NVRAM) does the router have?

There is 32768k bytes of NVRAM. your answers here.

How much Flash memory does the router have?

There is 3207167k bytes of flash memory. your answers here.

The show commands often provide multiple screens of outputs. Filtering the output allows a user to display certain sections of the output. To enable the filtering command, enter a pipe (|) character after a show command, followed by a filtering parameter and a filtering expression. You can match the output to the filtering statement by using the include keyword to display all lines from the output that contain the filtering expression. Filter the show version command, using show version | include register to answer the following question.

What is the boot process for the router on the next reload?

The register is 0x2102 your answers here.

### Display the startup configuration.

Use the show startup-config command on the router to answer the following questions.

How are passwords presented in the output?

They are encrypted. your answers here.

Use the show startup-config | section vty command.

What is the result of using this command?

It shows the configurations for the vty lines.

Type your answers here.

### Display the routing table on the router.

Use the show ip route command on the router to answer the following questions.

#### Questions:

What code is used in the routing table to indicate a directly connected network?

The C code is used. your answers here.

How many route entries are coded with a C code in the routing table?

There are three entries that have a C code. your answers here.

### Display a summary list of the interfaces on the router.

Use the show ip interface brief command on the router to answer the following question.

#### Question:

What command changed the status of the Gigabit Ethernet ports from administratively down to up?

The No Shutdown command was used. your answers here.

    - Use the show ipv6 int brief command to verify IPv6 settings on R1.

#### Question:

What is the meaning of the [up/up] part of the output?

Both the link-local and the ipv6 is up. your answers here.

    - On PC-B, change its configuration so that it no longer has a static IPv6 address. You may have to reboot the machine. Then, issue the ipconfig command on PC-B to examine the IPv6 configuration.

#### Questions:

What is the IPv6 address assigned to PC-B?

2001:db8:acad:1:7291:4374:f00:933f was assigned. your answers here.

What is the default gateway assigned to PC-B?

Fe80::1%23 was assigned. your answers here.

Issue a ping from PC-B to the R1 default gateway link local address. Was it successful?

Yes, it was successful. your answers here.

Issue a ping from PC-B to the R1 IPv6 unicast address 2001:db8:acad::1. Was it successful?

Yes, it was successful. your answers here.

# Reflection Questions

In researching a network connectivity issue, a technician suspects that an interface was not enabled. What show command could the technician use to troubleshoot this issue?

They should use the show ip interface brief command.your answers here.

In researching a network connectivity issue, a technician suspects that an interface was assigned an incorrect subnet mask. What show command could the technician use to troubleshoot this issue?

They could use the show ip interface command.

Paste your configurations here:

Using 2275 out of 33554432 bytes

!

! Last configuration change at 16:21:22 UTC Tue Jan 20 2026

! NVRAM config last updated at 10:10:39 UTC Tue Jan 20 2026

!

version 16.6

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname JCaldwellR1

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

security passwords min-length 12

enable secret 5 $1$i5M9$g7ATZ4S5DfsTD6zy7sjOI0

!

no aaa new-model

!

no ip domain lookup

ip domain name ccna-lab.com

!

!

!

login block-for 120 attempts 3 within 60

!

!

!

!

!

!

!

subscriber templating

!

!

!

!

!

ipv6 unicast-routing

!

!

multilink bundle-name authenticated

!

!

!

!

!

!

license udi pid ISR4321/K9 sn FLM2444000A

license boot level securityk9

diagnostic bootup level minimal

spanning-tree extend system-id

!

!

!

username SSHadmin secret 5 $1$SzLn$Q8l4pBQ4Bm9yvo5.zDZJR/

!

redundancy

mode none

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

interface Loopback0

ip address 10.0.0.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:2::1/64

!

interface GigabitEthernet0/0/0

ip address 192.168.0.1 255.255.255.0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD::1/64

!

interface GigabitEthernet0/0/1

ip address 192.168.1.1 255.255.255.0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

!

!

!

!

!

!

!

control-plane

!

banner motd ^CWarning! Unauthorized access prohibited^C

!

line con 0

exec-timeout 4 0

password 7 155608051729246569101A0C59

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

exec-timeout 4 0

password 7 114D1A0C0411044D451C1F1D62

login local

transport input ssh

line vty 5 15

exec-timeout 4 0

password 7 114D1A0C0411044D451C1F1D62

login local

transport input ssh

!

wsma agent exec

!

wsma agent config

!

wsma agent filesys

!

wsma agent notify

!

!

end