Net 1061 Switching, Routing, and Wireless Essentials

Name:

Lab:  #

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. If this is a self-grading packet tracer. Ensure you paste the screen shot of your score page at the bottom of the document. You will upload both this document and the pkt file regardless if it is self-grading or not. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

In this lab, I connected to a switch using telnet. Then, I configured SSH on the switch. Finally, I connected to it using SSH.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab I learned the difference between SSH and telnet. I learned how to configure SSH on a switch.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you learned from that assistance.

# Packet Tracer - Configure SSH

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 1 | 10.10.10.2 | 255.255.255.0 |
| PC1 | NIC | 10.10.10.10 | 255.255.255.0 |

# Objectives

Part 1: Secure Passwords

Part 2: Encrypt Communications

Part 3: Verify SSH Implementation

# Background

SSH should replace Telnet for management connections. Telnet uses insecure plain text communications. SSH provides security for remote connections by providing strong encryption of all transmitted data between devices. In this activity, you will secure a remote switch with password encryption and SSH.

# Instructions

## Secure Passwords

Using the command prompt on PC1, Telnet to S1. The user EXEC and privileged EXEC password is cisco.

Save the current configuration so that any mistakes you might make can be reversed by toggling the power for S1.

Show the current configuration and note that the passwords are in plain text. Enter the command that encrypts plain text passwords:

S1(config)# service password-encryption

Verify that the passwords are encrypted.

## Encrypt Communications

### Set the IP domain name and generate secure keys.

It is generally not safe to use Telnet, because data is transferred in plain text. Therefore, use SSH whenever it is available.

Configure the domain name to be netacad.pka.

Secure keys are needed to encrypt the data. Generate the RSA keys using a 1024 key length.

### Create an SSH user and reconfigure the VTY lines for SSH-only access.

Create an administrator user with cisco as the secret password.

Configure the VTY lines to check the local username database for login credentials and to only allow SSH for remote access. Remove the existing vty line password.

### Verify SSH Implementation

Exit the Telnet session and attempt to log back in using Telnet. The attempt should fail.

Attempt to log in using SSH. Type ssh and press Enter without any parameters to reveal the command usage instructions. Hint: The -l option is the letter “L”, not the number 1.

Upon successful login, enter privileged EXEC mode and save the configuration. If you were unable to successfully access S1, toggle the power and begin again at Part 1.

End of document