Net 1061 Switching, Routing, and Wireless Essentials

Name:

Lab:  #         7.4.2 Implement DHCPv4

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. You will also have to copy and paste your running configurations of each device configured to the bottom of this document. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

In this lab, first I created an address scheme and cabled all the devices. Then, I did the basic configurations on the switches and routers. Then, I configured the interfaces, VLANs, and trunk ports on the switches and routers. Then, I set up a DHCP server on the network. Finally, I verified connectivity.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned how to set up a DHCP server on a router. I learned how to set up a DHCP relay to allow the DHCP information to pass through.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you learned from that assistance.

Lab - Implement DHCPv4

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 10.0.0.1 | 255.255.255.252 | N/A N/A N/A N/A N/A |
| R1 | G0/0/1 | N/A | N/A |  |
| R1 | G0/0/1.100 | 192.168.1.1 | 255.255.255.192 |  |
| R1 | G0/0/1.200 | 192.168.1.65 | 255.255.255.224 |  |
| R1 | G0/0/1.1000 | N/A | N/A |  |
| R2 | G0/0/0 | 10.0.0.2 | 255.255.255.252 | N/A |
| R2 | G0/0/1 | 192.168.1.97 | 255.255.255.240 | N/A |
| S1 | VLAN 200 | 192.168.1.66 | 255.255.255.224 | 192.168.1.65 |
| S2 | VLAN 1 | 192.168.1.98 | 255.255.255.240 | 192.168.1.97 |
| PC-A | NIC | DHCP | DHCP | DHCP |
| PC-B | NIC | DHCP | DHCP | DHCP |

VLAN Table

| VLAN | Name | Interface Assigned |
| --- | --- | --- |
| 1 | N/A | S2: F0/18 |
| 100 | Clients | S1: F0/6 |
| 200 | Management | S1: VLAN 200 |
| 999 | Parking_Lot | S1: F0/1-4, F0/7-24, G0/1-2 |
| 1000 | Native | N/A |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure and verify two DHCPv4 Servers on R1

Part 3: Configure and verify a DHCP Relay on R2

# Background / Scenario

The Dynamic Host Configuration Protocol (DHCP) is a network protocol that lets network administrators manage and automate the assignment of IP addresses. Without DHCP for IPv4, the administrator must manually assign and configure IP addresses, preferred DNS servers, and default gateways. As the network grows in size, this becomes an administrative problem when devices are moved from one internal network to another.

In this scenario, the company has grown in size, and the network administrators can no longer assign IP addresses to devices manually. Your job is to configure the R1 router to assign IPv4 addresses on two different subnets.

Note: The routers used with CCNA hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4

(universalk9 image). The switches used in the labs are Cisco Catalyst 2960s with Cisco IOS Release 15.2(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Ensure that the routers and switches have been erased and have no startup configurations. If you are unsure contact your instructor.

# Required Resources

- 2 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

- 2 Switches (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable)

- 2 PCs (Windows with a terminal emulation program, such as Tera Term)

- Console cables to configure the Cisco IOS devices via the console ports

- Ethernet cables as shown in the topology

# Instructions Part 1: Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the PC hosts and switches.

## Step 1: Establish an addressing scheme

Subnet the network 192.168.1.0/24 to meet the following requirements:

- One subnet, “Subnet A”, supporting 58 hosts (the client VLAN at R1). Subnet A:

192.68.1.0/26 255.255.255.192 your answers here.

Record the first IP address in the Addressing Table for R1 G0/0/1.100.

- One subnet, “Subnet B”, supporting 28 hosts (the management VLAN at R1).  Subnet B: 192.168.1.64/27 255.255.255.224

Type your answers here.

Record the first IP address in the Addressing Table for R1 G0/0/1.200. Record the second IP address in the Address Table for S1 VLAN 200 and enter the associated default gateway.

- One subnet, “Subnet C”, supporting 12 hosts (the client network at R2).

Subnet C: 192.168.1.96/28 255.255.255.240

Record the first IP address in the Addressing Table for R2 G0/0/1.  Record the second IP address in the Address Table for S2 VLAN 1 and enter the associated default gateway.

Type your answers here.

192.168.1.97 255.255.255.240.  192.168.1.98 255.255.255.240.   192.168.1.97.

Step 2: Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

Step 3: Configure basic settings for each router.

- Assign a device name to the router.

Open configuration window

- Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

- Assign class as the privileged EXEC encrypted password.

- Assign cisco as the console password and enable login.

- Assign cisco as the VTY password and enable login.

- Encrypt the plaintext passwords.

- Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

- Save the running configuration to the startup configuration file.

- Set the clock on the router to today’s time and date.

Note: Use the question mark (?) to help with the correct sequence of parameters needed to execute this command.

## Step 4: Configure Inter-VLAN Routing on R1

- Activate interface G0/0/1 on the router.

- Configure sub-interfaces for each VLAN as required by the IP addressing table. All sub-interfaces use 802.1Q encapsulation and are assigned the first usable address from the IP address pool you have calculated. Ensure the sub-interface for the native VLAN does not have an IP address assigned. Include a description for each sub-interface.

- Verify the sub-interfaces are operational.

## Step 5: Configure G0/0/1 on R2, then G0/0/0 and static routing for both routers

- Configure G0/0/1 on R2 with the first IP address of Subnet C you calculated earlier.

- Configure interface G0/0/0 for each router based on the IP Addressing table above.

- Configure a default route on each router pointed to the IP address of G0/0/0 on the other router.

- Verify static routing is working by pinging R2’s G0/0/1 address from R1.

- Save the running configuration to the startup configuration file.

Close configuration window

Step 6: Configure basic settings for each switch.

- Assign a device name to the switch.

Open configuration window

- Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

- Assign class as the privileged EXEC encrypted password.

- Assign cisco as the console password and enable login.

- Assign cisco as the VTY password and enable login.

- Encrypt the plaintext passwords.

- Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

- Save the running configuration to the startup configuration file.

- Set the clock on the switch to today’s time and date.

Note: Use the question mark (?) to help with the correct sequence of parameters needed to execute this command.

- Copy the running configuration to the startup configuration.

Step 7: Create VLANs on S1.

Note: S2 is only configured with basic settings.

- Create and name the required VLANs on switch 1 from the table above.

- Configure and activate the management interface on S1 (VLAN 200) using the second IP address from the subnet calculated earlier. Additionally, set the default gateway on S1.

- Configure and activate the management interface on S2 (VLAN 1) using the second IP address from the subnet calculated earlier. Additionally, set the default gateway on S2

- Assign all unused ports on S1 to the Parking_Lot VLAN, configure them for static access mode, and administratively deactivate them. On S2, administratively deactivate all the unused ports.

Note: The interface range command is helpful to accomplish this task with as few commands as necessary.

Close configuration window

Open configuration window

Close configuration window

Step 8: Assign VLANs to the correct switch interfaces.

- Assign used ports to the appropriate VLAN (specified in the VLAN table above) and configure them for static access mode.

Open configuration window

- Verify that the VLANs are assigned to the correct interfaces.

Question:

Why is interface F0/5 listed under VLAN 1?

It hasn’t been assigned to another vlan.

Type your answers here.

Step 9: Manually configure S1’s interface F0/5 as an 802.1Q trunk.

- Change the switchport mode on the interface to force trunking.

- As a part of the trunk configuration, set the native VLAN to 1000.

- As another part of trunk configuration, specify that VLANs 100, 200, and 1000 are allowed to cross the trunk.

- Save the running configuration to the startup configuration file.

- Verify trunking status.

Question:

At this point, what IP address would the PC’s have if they were connected to the network using DHCP?

None, DHCP is not configured yet.

Type your answers here.

Close configuration window

# Part 2: Configure and verify two DHCPv4 Servers on R1

In Part 2, you will configure and verify a DHCPv4 Server on R1. The DHCPv4 server will service two subnets, Subnet A and Subnet C.

## Step 1: Configure R1 with DHCPv4 pools for the two supported subnets. Only the DHCP Pool for subnet A is given below

- Exclude the first five useable addresses from each address pool.

Open configuration window

- Create the DHCP pool (Use a unique name for each pool).

- Specify the network that this DHCP server is supporting.

- Configure the domain name as ccna-lab.com

- Configure the appropriate default gateway for each DHCP pool.

- Configure the lease time for 2 days 12 hours and 30 minutes.

- Next, configure the second DHCPv4 Pool using the pool name R2_Client_LAN and the calculated network, default-router and use the same domain name and lease time from the previous DHCP pool.

## Step 2: Save your configuration

Save the running configuration to the startup configuration file.

Close configuration window

## Step 3: Verify the DHCPv4 Server configuration

- Issue the command show ip dhcp pool to examine the pool details.

- Issue the command show ip dhcp bindings to examine established DHCP address assignments.

- Issue the command show ip dhcp server statistics to examine DHCP messages.

## Step 4: Attempt to acquire an IP address from DHCP on PC-A

- Open a command prompt on PC-A and issue the command ipconfig /renew.

- Once the renewal process is complete, issue the command ipconfig to view the new IP information.

- Test connectivity by pinging R1’s G0/0/1 interface IP address.

# Part 3: Configure and verify a DHCP Relay on R2

In Part 3, you will configure R2 to relay DHCP requests from the local area network on interface G0/0/1 to the DHCP server (R1).

## Step 1: Configure R2 as a DHCP relay agent for the LAN on G0/0/1

- Configure the ip helper-address command on G0/0/1 specifying R1’s G0/0/0 IP address.

Open configuration window

- Save your configuration.

Close configuration window

## Step 2: Attempt to acquire an IP address from DHCP on PC-B

- Open a command prompt on PC-B and issue the command ipconfig /renew.

- Once the renewal process is complete, issue the command ipconfig to view the new IP information.

- Test connectivity by pinging R1’s G0/0/1 interface IP address.

- Issue the show ip dhcp binding on R1 to verify DHCP bindings.

- Issue the show ip dhcp server statistics on R1 and R2 to verify DHCP messages.

End of document

Building configuration...

Current configuration : 5285 bytes

!

! Last configuration change at 11:23:32 UTC Thu Feb 12 2026

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname JCaldwellS1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$IHB/VO2u4FoBG8$aTNMMigLGNSgOLPt9avmB11uZYf0Wsr.wY7jVJX5PC2

!

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

!

!

!

!

!

!

crypto pki trustpoint TP-self-signed-3235714944

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3235714944

revocation-check none

rsakeypair TP-self-signed-3235714944

!

!

crypto pki certificate chain TP-self-signed-3235714944

certificate self-signed 01

3082022B 30820194 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33323335 37313439 3434301E 170D3236 30323132 31303434

34325A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D33 32333537

31343934 3430819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100F076 C4E07D8B CC3A2B28 30479BC0 9999A15B A9D93AB4 FC7944DA CE07D463

2FC7EB00 57B78CFD E791283A 6CBA5B7A 722A121E 8FF9B79D CDF99B7A 05071178

9EB01E37 214C825C A01D73CF C4C50621 64A70530 3277AF67 26238D7E 365C75E6

EB14E6D0 3D42F6AA C1BA39E0 8F0B4A96 34BB29A6 DBB71E17 5E0691E2 F7967297

FA6B0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 1488B115 66ACC9EA C38F923B 58B420C9 40D23727 57301D06

03551D0E 04160414 88B11566 ACC9EAC3 8F923B58 B420C940 D2372757 300D0609

2A864886 F70D0101 05050003 818100BD FD16FD49 C952FA0C 082FCB27 901F1048

9D6336B3 C3D637CC CD88F6A1 53053FF1 9559C99D D83A03A2 41DB77F1 0A414908

2FAD3024 001C11EB EFC6519E 2AED45A0 9069A238 D929E8A2 24A24B23 0518957E

FC18AE4B 037AE2FD AD881A30 F8FFF27F E66E50FA 5F05D15E 29BA5044 82D9E317

A6E54314 D62402C5 A71AA1E9 84985E

quit

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/2

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/3

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/4

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/5

switchport trunk allowed vlan 100,200,1000

switchport trunk native vlan 1000

switchport mode trunk

!

interface GigabitEthernet1/0/6

switchport access vlan 100

switchport mode access

!

interface GigabitEthernet1/0/7

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/8

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/9

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/10

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/11

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/12

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/13

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/14

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/15

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/16

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/17

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/18

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/19

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/20

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/21

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/22

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/24

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/25

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/26

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/27

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/28

switchport access vlan 999

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan200

ip address 192.168.1.66 255.255.255.224

!

ip default-gateway 192.168.1.65

ip http server

ip http secure-server

!

!

!

banner motd ^CCUnauthorized access is prohibited^C

!

line con 0

password 7 00071A150754

login

line vty 0 4

password 7 110A1016141D

login

transport input none

line vty 5 15

password 7 121A0C041104

login

transport input none

!

End

Building configuration...

Current configuration : 5096 bytes

!

! Last configuration change at 16:50:53 UTC Thu Feb 12 2026

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname JCaldwellS2

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$vWRzIgz/h1Bejp$W.YVL4UzPqQZK85HoQ1Jj5/qPdknwLmxfPc4A0Ew.z2

!

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

!

!

!

!

!

!

crypto pki trustpoint TP-self-signed-3069817600

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3069817600

revocation-check none

rsakeypair TP-self-signed-3069817600

!

!

crypto pki certificate chain TP-self-signed-3069817600

certificate self-signed 01

3082022B 30820194 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33303639 38313736 3030301E 170D3236 30323132 31353337

30395A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D33 30363938

31373630 3030819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100A424 5267B10C E7A58104 0235F7BC E33EF897 6ED5369A DAAB6008 FFFF2668

538BA96C 60E09337 10CCAE28 6D5122BC D18F795A 8BDFF17A 5D9DF731 66B924D7

55040C40 B26E31CC 27A3E37E 28788674 428F9D7A 536966E3 4BA97EAC 45719F09

6E5F3345 8F77B992 81C53DBF 37657D22 8BA937C7 F47CF2DB 2B9C0A02 951790A7

040D0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 147E5D8F 9FC5CD24 9CF54BBF 93DE2384 A3A1B624 BD301D06

03551D0E 04160414 7E5D8F9F C5CD249C F54BBF93 DE2384A3 A1B624BD 300D0609

2A864886 F70D0101 05050003 81810015 7D5AA1D3 D09ACB74 D9E0D87E CBFF69D5

8E70DC94 74E09FB9 68D23B39 886751B5 8870F30A D6C05DA8 133541AF 30424346

C6591B4E 7AA0208F 77784127 9E44D0E1 6EA10785 D7C237C3 B7A763A2 65627990

20CA494C 21D7FB66 9DB16EEB 1A139FF5 F75A4372 912D1B66 DABC60FC 56C80516

3B95AD77 81BB0759 01188EEB 47D398

quit

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/2

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/3

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/4

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/7

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/8

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/9

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/10

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/11

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/12

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/13

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/14

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/15

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/16

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/17

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/20

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/21

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/22

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/24

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/25

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/26

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/27

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/28

switchport access vlan 999

switchport mode access

shutdown

!

interface Vlan1

ip address 192.168.1.98 255.255.255.240

shutdown

!

ip default-gateway 192.168.1.97

ip http server

ip http secure-server

!

!

!

banner motd ^CCUnauthorized access is prohibited^C

!

line con 0

password 7 01100F175804

login

line vty 0 4

password 7 01100F175804

login

transport input none

line vty 5 15

password 7 070C285F4D06

login

transport input none

!

End

Current configuration : 6768 bytes

!

! Last configuration change at 16:57:31 UTC Thu Feb 12 2026

!

version 17.3

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service call-home

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname JCaldwellR1

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$Isy40wU1QVd8XU$bfw/U7sz1gINT.uhkcB.pDNCQ.KUmld3g9Mxihxh40w

!

no aaa new-model

!

!

!

!

!

!

!

no ip domain lookup

ip dhcp excluded-address 192.168.1.1 192.168.1.5

!

ip dhcp pool R1_Client_Lan

network 192.168.1.0 255.255.255.192

default-router 192.168.1.1

domain-name ccna-lab.com

lease 2 12 30

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

multilink bundle-name authenticated

no device-tracking logging theft

!

!

!

crypto pki trustpoint TP-self-signed-2037616329

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2037616329

revocation-check none

rsakeypair TP-self-signed-2037616329

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

!

crypto pki certificate chain TP-self-signed-2037616329

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32303337 36313633 3239301E 170D3236 30323132 31363038

34375A17 0D333630 32313231 36303834 375A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 30333736

31363332 39308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100A08B D54E6116 0CFB0E5D AE58FCB7 FDC0B968 886E2B58 079DC977

F76100F6 79FAD02C 2DC81D33 67DEE925 7B48911E 4E1AECE1 ACC44665 8EAA1F19

638108D9 FA7464B4 E7D54AFD 1DF28761 DBB8C5F4 CEE89F49 2D4F4A67 1E2F415A

4B5793BA D3F3DE13 F8953423 EDB8E286 50573B85 85DDABFC 2C267B83 1395A2D6

CDEBF612 4BD1C3B0 C5CB56BC 4AB6448C D29EB538 702853B9 3EF55B93 B91A5F24

98823282 6E0B0DDB FC719316 2D73744D 5EDC9BEC 7608FEC5 B8EF8B45 118981B0

6E4334E9 4E86B378 F8A9F76A 6EB603CB CDD8B7E8 5578A055 76B98372 E320C468

93834A75 5F987047 25BA1069 7DFC95A7 F6927140 6C84F11E 503B90E4 919BDE75

3D53CA22 8A790203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14B207A9 046C855D F905E510 A842EDDE FEC184E5

D5301D06 03551D0E 04160414 B207A904 6C855DF9 05E510A8 42EDDEFE C184E5D5

300D0609 2A864886 F70D0101 05050003 82010100 14825D1B BD330615 405BC3DC

574A271A C38CD30B AD70FCCB CFA6AD9F D63E56C4 C931E7E5 F6D1D68B 1A26E0C0

BE621667 6EF916E6 0991D6B1 9BBEE95C 33F1C78C 2A4F78E2 4B1F967A 22862E0D

7DAFC4C1 9233CE24 C029AADF BD2E4514 C73AC5A9 CA4698DE 2C136FAF FD16E681

3CDCC18E AB090A17 A8709B99 41F8496B 3E27C079 E031217B 154F1AA3 AD16B96C

549A0DA9 3985E619 50D1A998 1D854374 44A636C4 838EA619 57228089 50CB9A65

A2551358 D43577EC EC17CE8F C59C56A2 9D1F5B74 6A311EE2 5B7CCD3F 2F253DF4

873F3246 81BF94E6 C36418E2 AA2F734E 057FA239 FC7573DE 4BB5F9BF 0217A15D

309B9A29 89D03AE0 49F44BF3 FE7E0DB0 43125FF3

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

!

!

license udi pid ISR4321/K9 sn FLM254904JP

memory free low-watermark processor 69075

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 10.0.0.1 255.255.255.252

negotiation auto

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1.100

description vlan100

encapsulation dot1Q 100

ip address 192.168.1.1 255.255.255.192

!

interface GigabitEthernet0/0/1.200

description vlan200

encapsulation dot1Q 200

ip address 192.168.1.65 255.255.255.224

!

interface GigabitEthernet0/0/1.1000

description vlan1000

encapsulation dot1Q 1000 native

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

ip http server

ip http authentication local

ip http secure-server

ip forward-protocol nd

ip tftp source-interface GigabitEthernet0

ip route 0.0.0.0 0.0.0.0 10.0.0.2

!

!

!

!

!

!

control-plane

!

banner motd ^CCUnauthorized access is prohibited^C

!

line con 0

password 7 121A0C041104

login

stopbits 1

line aux 0

stopbits 1

line vty 0 4

password 7 1511021F0725

login

transport input ssh

line vty 5 15

password 7 060506324F41

login

transport input ssh

!

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

!

!

!

!

!

!

End

Current configuration : 6644 bytes

!

! Last configuration change at 17:30:00 UTC Thu Feb 12 2026

!

version 17.3

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service call-home

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname JCaldwellR2

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$o77DkDXAu4oRik$n5cnjvjNr7Q53ik6qL2NqAqntdcI2ToOQOKv0dmyLhw

!

no aaa new-model

!

!

!

!

!

!

!

no ip domain lookup

ip dhcp excluded-address 192.168.1.97 192.168.1.101

!

ip dhcp pool R2_Client_Lan

network 192.168.1.96 255.255.255.240

default-router 192.168.1.97

domain-name ccna-lab.com

lease 2 12 30

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

multilink bundle-name authenticated

no device-tracking logging theft

!

!

!

crypto pki trustpoint TP-self-signed-94484111

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-94484111

revocation-check none

rsakeypair TP-self-signed-94484111

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

!

crypto pki certificate chain TP-self-signed-94484111

certificate self-signed 01

3082032C 30820214 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

2F312D30 2B060355 04031324 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 39343438 34313131 301E170D 32363032 31323136 31353231

5A170D33 36303231 32313631 3532315A 302F312D 302B0603 55040313 24494F53

2D53656C 662D5369 676E6564 2D436572 74696669 63617465 2D393434 38343131

31308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201 0A028201

0100A623 6F85CDEC EFD078FE 44A17AF4 2AC462BB A65B57A9 02F9D320 87BBF073

D8B188E8 CE6BCEEE 31C1F740 C828AA76 924BA7F7 9BAEDD8B CEB92947 7E5941E1

F1A338E1 5256DB39 A551F81C 95CD4923 67925C6A 9B4A7F3C CD039AB2 548F467E

4D07C38A E3A007D8 1DF45BB7 E3B0F302 FAD64D01 46640480 F149C631 C9F35529

318BE5E4 7FB02555 F518722C 78497579 3359326A AA10C539 A41CC71E D1750E5D

8B681F6D 268BC7DE 82385410 7854E0EC E7308513 CCC45DF5 3B982259 D5D1065C

8C254830 D1D0120F 47CAD313 6B5B0FB3 6D7EED5D DBC39803 E54AC5E5 91A24233

8E8E40F0 F7829C24 DA751F3C F3425E0C ABD9B334 B1E1108D C9DE6A87 28F9D15B

D22B0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 14FA6D34 0EEB4CE6 DC84700C 05214B24 EBE08936 B0301D06

03551D0E 04160414 FA6D340E EB4CE6DC 84700C05 214B24EB E08936B0 300D0609

2A864886 F70D0101 05050003 82010100 65263E3B 2E8803C3 0ED5F6BC D8CD8802

1F0DF22A 564879F7 DBAAA3B2 0044B312 F2684201 72DAFDEA 2458277F 5753BCA4

981F2ECE B6DD50F7 25262340 2227C3D5 53B8BB50 3ABDD321 C289A21A 3FA57073

EB2BEA4B 143AADB1 6322FB55 6AD4B4A8 8F555439 13BDE40B 64928464 9553C5F2

A8AB8739 790E6146 F7B25A44 7C45EEC6 47B0B0E7 824C24F9 D4B52B4F CEA92B86

1AEFDA0B 40F47E43 64482A56 2A8C2735 60CFCA01 377F7284 9421680A A27BAB68

44C4813C 4085B66D 1026FEB0 738B1BEF 61185A4E CDED9E61 BC963CE1 00BB2550

E9E6ED35 736693DD 1330AAB8 25758ECE C3A1CC30 34A62EF3 6BD44A0B D56EB09B

38C53EC3 9B808088 D62C85E9 B1EF251D

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

!

!

license udi pid ISR4321/K9 sn FLM254904MP

memory free low-watermark processor 69075

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 192.168.1.97 255.255.255.240

ip helper-address 10.0.0.1

negotiation auto

!

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

!

interface Serial0/2/0

no ip address

shutdown

!

interface Serial0/2/1

no ip address

shutdown

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

ip http server

ip http authentication local

ip http secure-server

ip forward-protocol nd

ip tftp source-interface GigabitEthernet0

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

!

!

!

!

!

control-plane

!

banner motd ^CCUnauthorized access is prohibited^C

!

line con 0

password 7 060506324F41

login

stopbits 1

line aux 0

stopbits 1

line vty 0 4

password 7 104D000A0618

login

transport input ssh

line vty 5 15

password 7 070C285F4D06

login

transport input ssh

!

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

!

!

!

!

!

!

End