Net 1061 Switching, Routing, and Wireless Essentials

Name:

Lab:  #        3.6.2 Implement VLANs and Trunking

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. You will also have to copy and paste your running configurations of each device configured to the bottom of this document. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

In this lab, first, I cabled all the devices used. Then, I configured the switches and PC hosts. Then, I set up the VLANs for both switches and assigned the correct ports to them. Then, I configured the trunk port for the switches. Finally, I verified connectivity.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned how to set up VLANs and how to configure them on switches. I learned how switches send VLAN information between devices and how there is a need for a router.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you learned from that assistance.

# Lab - Implement VLANs and Trunking

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 10 | 192.168.10.11 | 255.255.255.0 |
| S1 | VLAN 20 | 192.168.20.11 | 255.255.255.0 |
| S1 | VLAN 30 | 192.168.30.11 | 255.255.255.0 |
| S2 | VLAN 10 | 192.168.10.12 | 255.255.255.0 |
| PC-A | NIC | 192.168.20.13 | 255.255.255.0 |
| PC-B | NIC | 192.168.30.13 | 255.255.255.0 |

# VLAN Table

| VLAN | Name | Interface Assigned |
| --- | --- | --- |
| 10 | Management | S1: VLAN 10 S2: VLAN 10 |
| 20 | Sales | S1: VLAN 20 and F0/6 |
| 30 | Operations | S1: VLAN 30 S2: F0/18 |
| 999 | ParkingLot | S1: F0/2-5, F0/7-24, G0/1-2 S2: F0/2-17, F0/19-24, G0/1-2 |
| 1000 | Native | N/A |

# Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Create VLANs and Assign Switch Ports

Part 3: Configure an 802.1Q Trunk between the Switches

# Background / Scenario

Modern switches use virtual local-area networks (VLANs) to improve network performance by separating large Layer 2 broadcast domains into smaller ones. VLANs address scalability, security, and network management. In general, VLANs make it easier to design a network to support the goals of an organization. Communication between VLANs requires a device operating at Layer 3 of the OSI model.

VLAN trunks are used to span VLANs across multiple devices. Trunks allow the traffic from multiple VLANs to travel over a single link, while keeping the VLAN identification and segmentation intact.

In this lab, you will create VLANs on both switches in the topology, assign VLANs to switch access ports, verify that VLANs are working as expected and create VLAN trunks between the two switches.

Note: The switches used with CCNA hands-on labs are Cisco Catalyst 2960s with Cisco IOS Release 15.2(2) (lanbasek9 image). Other switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Ensure that the switches have been erased and have no startup configurations. If you are unsure contact your instructor.

# Required Resources

2 Switches (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable)

2 PCs (Windows with a terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

# Instructions

## Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the PC hosts and switches.

### Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

### Configure basic settings for each switch.

Open configuration window

Console into the switch and enable privileged EXEC mode.

Assign a device name to the switch.

Disable DNS lookup.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plaintext passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Copy the running configuration to the startup configuration.

Close configuration window

### Configure PC hosts.

Refer to the Addressing Table for PC host address information.

## Create VLANs and Assign Switch Ports

In Part 2, you will create VLANs as specified in the table above on both switches. You will then assign the VLANs to the appropriate interface. The show vlan brief command is used to verify your configuration settings. Complete the following tasks on each switch.

### Create VLANs on both switches.

Open configuration window

Create and name the required VLANs on each switch from the table above.

Configure the management interface on each switch using the IP address information in the Addressing Table.

Assign all unused ports on the switch to the ParkingLot VLAN, configure them for static access mode, and administratively deactivate them.

### Assign VLANs to the correct switch interfaces.

Assign used ports to the appropriate VLAN (specified in the VLAN table above) and configure them for static access mode.

Verify that the VLANs are assigned to the correct interfaces.

Close configuration window

## Configure an 802.1Q Trunk Between the Switches

In Part 3, you will manually configure interface F0/1 as a trunk.

### Manually configure trunk interface F0/1.

Open configuration window

Change the switchport mode on interface F0/1 to force trunking. Make sure to do this on both switches.

Set the native VLAN to 1000 on both switches.

As another part of trunk configuration, specify that only VLANs 10, 20, 30, and 1000 are allowed to cross the trunk.

Issue the show interfaces trunk command to verify trunking ports, the native VLAN and allowed VLANs across the trunk.

Close configuration window

### Verify connectivity.

Verify connectivity within a VLAN. For example, PC-A should be able to ping S1 VLAN 20 successfully.

#### Question:

Were the pings from PC-B to S2 successful? Explain.

No, the pings aren’t going to work without a router because the two are on different networks.

Paste your configurations here:

Building configuration...

Current configuration : 3757 bytes

!

! Last configuration change at 11:36:20 UTC Thu Jan 29 2026

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname JCaldwellS1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$1BGk2c.YQHyVyO$TXdqCuZ7qd.7bMloDnItTgnyrT2nAiU9Njk7fnYc0lE

!

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

!

!

!

!

!

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport trunk allowed vlan 10,20,30,1000

switchport trunk native vlan 1000

switchport mode trunk

!

interface GigabitEthernet1/0/2

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/3

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/4

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/5

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/6

switchport access vlan 20

switchport mode access

!

interface GigabitEthernet1/0/7

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/8

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/9

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/10

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/11

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/12

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/13

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/14

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/15

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/16

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/17

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/18

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/19

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/20

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/21

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/22

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/24

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/25

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/26

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/27

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/28

switchport access vlan 999

switchport mode access

shutdown

!

interface Vlan1

no ip address

!

interface Vlan10

ip address 192.168.10.11 255.255.255.0

!

interface Vlan20

ip address 192.168.20.11 255.255.255.0

!

interface Vlan30

ip address 192.168.30.11 255.255.255.0

!

ip http server

ip http secure-server

!

!

!

banner motd ^CUnauthorized access is prohibited^C

!

line con 0

password 7 030752180500

login

line vty 0 4

password 7 030752180500

login

transport input none

line vty 5 15

password 7 030752180500

login

transport input none

!

End

Building configuration...

Current configuration : 3649 bytes

!

! Last configuration change at 16:24:24 UTC Thu Jan 29 2026

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname JCaldwellS2

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$3/oFJzaNOxT8vJ$eXJNGNME5olLnjQCOJmBOcRAleB5HQ72wTrmw5z5O/o

!

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

!

!

!

!

!

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface GigabitEthernet1/0/1

switchport trunk allowed vlan 10,20,30,1000

switchport trunk native vlan 1000

switchport mode trunk

!

interface GigabitEthernet1/0/2

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/3

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/4

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/5

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/6

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/7

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/8

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/9

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/10

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/11

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/12

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/13

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/14

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/15

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/16

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/17

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/18

switchport access vlan 30

switchport mode access

!

interface GigabitEthernet1/0/19

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/20

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/21

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/22

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/24

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/25

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/26

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/27

switchport access vlan 999

switchport mode access

shutdown

!

interface GigabitEthernet1/0/28

switchport access vlan 999

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.12 255.255.255.0

!

ip http server

ip http secure-server

!

!

!

banner motd ^CUnauthorized access is prohibited^C

!

line con 0

password 7 030752180500

login

line vty 0 4

password 7 030752180500

login

transport input none

line vty 5 15

password 7 030752180500

login

transport input none

!

End

Type your answers here.

End of document