Net 1061 Switching, Routing, and Wireless Essentials

Name:                                     Lab:  #

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. Ensure you paste the screenshot of your score page at the bottom of the document. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences, summarize the work you completed during the lab.

In this lab, I created an addressing scheme for subnets. Then I configured the PC’s ip information. Then I did basic configurations for the Router. Then I did the same for all the switches. Finally, I verified the connection.

2. Using complete sentences, describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I relearned all the main skills I learned in the previous semester.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

When I went to enter the ipv6 for the PCs, it wouldn’t register. The ipv6 addresses worked for the router interfaces.

2. Using complete sentences, describe how you solved your problem(s).

I didn’t solve my problem.

3. Using complete sentences, explain if you needed any assistance with the lab; then list what you learned from that assistance.

Packet Tracer – Skills Integration Challenge

Topology

Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
|  |  | IPv6 Address/Prefix | IPv6 Link-local |  |
| R1 | G0/0 | 192.168.0.1 | 255.255.255.128 | N/A |
|  |  | 2001:DB8:ACAD::1/64 | FE80::1 | N/A |
|  | G0/1 | 192.168.0.129 | 255.255.255.192 | N/A |
|  |  | 2001:DB8:ACAD:1::1/64 | FE80::1 | N/A |
|  | G0/2 | 192.168.0.193 | 255.255.255.224 | N/A |
|  |  | 2001:DB8:ACAD:2::1/64 | FE80::1 | N/A |
|  | S0/0/1 | 172.16.1.2 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:2::1/64 | FE80::1 | N/A |
| Central | S0/0/0 | 209.165.200.226 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:1::1/64 | FE80::2 | N/A |
|  | S0/0/1 | 172.16.1.1 | 255.255.255.252 | N/A |
|  |  | 2001:DB8:2::2/64 | FE80::2 | N/A |
| S1 | VLAN 1 | 192.168.0.2 | 255.255.255.128 | 192.168.0.1 |
| S2 | VLAN 1 | 192.168.0.130 | 255.255.255.192 | 192.168.0.129 |
| S3 | VLAN 1 | 192.168.0.194 | 255.255.255.224 | 192.168.0.193 |
| Staff | NIC | 192.168.0.126 | 255.255.255.128 | 192.168.0.1 |
|  |  | 2001:DB8:ACAD::2/64 | FE80::2 | FE80::1 |
| Sales | NIC | 192.168.0.190 | 255.255.255.192 | 192.168.0.129 |
|  |  | 2001:DB8:ACAD:1::2/64 | FE80::2 | FE80::1 |
| IT | NIC | 192.168.0.222 | 255.255.255.224 | 192.168.0.193 |
|  |  | 2001:DB8:ACAD:2::2/64 | FE80::2 | FE80::1 |
| Web | NIC | 64.100.0.3 | 255.255.255.248 | 64.100.0.1 |
|  |  | 2001:DB8:CAFE::3/64 | FE80::2 | FE80::1 |

Scenario / Background

The router Central, ISP cluster and the Web server are completely configured. You have been tasked with creating a new IPv4 addressing scheme that will accommodate 4 subnets using 192.168.0.0/24 network. The IT department requires 25 hosts. The Sales department needs 50 hosts. The subnet for the rest of the staff requires 100 hosts. A Guest subnet will be added in the future to accommodate 25 hosts. You are also tasked with finishing the basic security settings and interface configurations on R1. Furthermore, you will configure the SVI interface and basic security setting on switches S1, S2 and S3.

Requirements

IPv4 Addressing

Create subnets that meet the host requirements using 192.168.0.0/24.

Staff: 100 hosts

Sales: 50 hosts

IT: 25 hosts

Guest network to be added later: 25 hosts

Document the assigned IPv4 addresses in the Addressing Table.

Record the subnet for Guest network:_192.168.0.224__________________________

PC Configurations

Configure the assigned IPv4 address, subnet mask, and default gateway settings on the Staff, Sales and IT PCs using your addressing scheme.

Assign IPv6 unicast and link local addresses and default gateway to the Staff, Sales, and IT networks according to the Addressing Table.

R1 Configurations

Configure the device name according to the Addressing Table.

Disable DNS lookup.

Assign Ciscoenpa55 as the encrypted privileged EXEC mode password.

Assign Ciscoconpa55 as the console password and enable login.

Require that a minimum of 10 characters be used for all passwords.

Encrypt all plaintext passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited. Make sure to include the word Warning in the banner.

Configure all the Gigabit Ethernet interfaces.

Configure the IPv4 addresses according to your addressing scheme.

Configure the IPv6 addresses according to the Addressing Table.

Configure SSH on R1:

Set the domain name to CCNA-lab.com

Generate a 1024-bit RSA key.

Configure the VTY lines for SSH access.

Use the local user profiles for authentication.

Create a user Admin1 with a privilege level of 15 using the encrypted password for Admin1pa55.

Configure the console and VTY lines to log out after five minutes of inactivity.

Block anyone for three minutes who fails to log in after four attempts within a two-minute period.

Switch Configurations

Configure the device name according to the Addressing Table.

Configure the SVI interface with the IPv4 address and subnet mask according your addressing scheme.

Configure the default gateway.

Disable DNS lookup.

Assign Ciscoenpa55 as the encrypted privileged EXEC mode password.

Assign Ciscoconpa55 as the console password and enable login.

Configure the console and VTY lines to log out after five minutes of inactivity.

Encrypt all plaintext passwords.

Verify Connectivity

Using the web browser from Staff, Sales, and IT PCs, navigate to www.cisco.pka.

Using the web browser from Staff, Sales, and IT PCs, navigate to www.cisco6.pka.

All PCs should be able to ping all the devices.