Net 1060   Introduction to Networks              Lab:  #   14.8.1

Name:

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. Ensure you paste the screen shot of your score page at the bottom of the document. Even if this does not let you see your grade, still take a screen shot of the score page showing congratulations “your name” you have completed the exercise is pasted at the bottom of this document. You will then need to upload both this word document and your packet tracer file to the assignments link within Netacad. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

First I did a ping command and analyzed the traffic and pdus generated. Then, I searched a website and analyzed the traffic. Then, I did an ftp command and analyzed the pdus it created. Then, I did a dns lookup and analyzed the connection. Finally, I sent an email and analyzed the traffic.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned about how to analyze the traffic that different actions generate and compare them. I also learned more about the tcp and udp protocols and how packets travel.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you    learned from that assistance.

# Packet Tracer - TCP and UDP Communications

# Objectives

Part 1: Generate Network Traffic in Simulation Mode

Part 2: Examine the Functionality of the TCP and UDP Protocols

# Background

This simulation activity is intended to provide a foundation for understanding TCP and UDP in detail. Packet Tracer simulation mode provides you the ability to view the state of different PDUs as they travel through the network.

Packet Tracer Simulation mode enables you to view each of the protocols and the associated PDUs. The steps outlined below lead you through the process of requesting network services using various applications that are available on a client PC. You will explore the functionality of the TCP and UDP protocols, multiplexing, and the function of port numbers in determining which local application requested the data or is sending the data.  Packet Tracer will not score this activity.

# Instructions

## Generate Network Traffic in Simulation Mode and View Multiplexing

### Generate traffic to populate Address Resolution Protocol (ARP) tables.

Perform the following task to reduce the amount of network traffic viewed in the simulation.

Click MultiServer and click the Desktop tab > Command Prompt.

Enter the ping -n 1 192.168.1.255 command. You are pinging the broadcast address for the client LAN. The command option will send only one ping request rather than the usual four. This will take a few seconds as every device on the network responds to the ping request from MultiServer.

Close the MultiServer window.

### Generate web (HTTP) traffic.

Switch to Simulation mode.

Click HTTP Client and open the Web Browser from the desktop.

In the URL field, enter 192.168.1.254 and click Go. Envelopes (PDUs) will appear in the topology window.

Minimize, but do not close, the HTTP Client configuration window.

### Generate FTP traffic.

Click FTP Client and open the Command Prompt from the desktop

Enter the ftp 192.168.1.254 command. PDUs will appear in the simulation window.

Minimize, but do not close, the FTP Client configuration window.

### Generate DNS traffic.

Click DNS Client and open the Command Prompt.

Enter the nslookup multiserver.pt.ptu command. A PDU will appear in the simulation window.

Minimize, but do not close, the DNS Client configuration window.

### Generate Email traffic.

Click E-Mail Client and open the E Mail tool from the Desktop.

Click Compose and enter the following information:

To: user@multiserver.pt.ptu

Subject: personalize the subject line

E-Mail Body: personalize the Email

Click Send.

Minimize, but do not close, the E-Mail Client configuration window.

### Verify that the traffic is generated and ready for simulation.

There should now be PDU entries in the simulation panel for each of the client computers.

### Examine multiplexing as the traffic crosses the network.

You will now use the Capture/Forward button in the Simulation Panel to observe the different protocols travelling on the network.

Note: The Capture/Forward button ‘ >| ‘ is a small arrow pointing to the right with a vertical bar next to it.

Click Capture/Forward once. All of the PDUs travel to the switch.

Click Capture/Forward six times and watch the PDUs from the different hosts as they travel on the network. Note that only one PDU can cross a wire in each direction at any given time.

#### Questions:

What is this called?

This is called multiplexing.pe your answers here.

A variety of PDUs appears in the event list in the Simulation Panel. What is the meaning of the different colors?

The different colors represent different protocols.Type your answers here.

## Examine Functionality of the TCP and UDP Protocols

### Examine HTTP traffic as the clients communicate with the server.

Click Reset Simulation.

Filter the traffic that is currently displayed to only HTTP and TCP PDUs. To filter the traffic that is currently displayed:

Click Edit Filters and toggle the Show All/None button.

Select HTTP and TCP. Click the red “x” in the upper right-hand corner of the Edit Filters box to close it. Visible Events should now display only HTTP and TCP PDUs.

Open the browser on HTTP Client and enter 192.168.1.254 in the URL field. Click Go to connect to the server over HTTP. Minimize the HTTP Client window.

Click Capture/Forward until you see a PDU appear for HTTP. Note that the color of the envelope in the topology window matches the color code for the HTTP PDU in the Simulation Panel.

#### Question:

Why did it take so long for the HTTP PDU to appear?

It took a second to appear because it had to connect with the server.Type your answers here.

Click the PDU envelope to show the PDU details. Click the Outbound PDU Details tab and scroll down to the second to the last section.

#### Questions:

What is the section labeled?

It is labeled TCP.Type your answers here.

Are these communications considered to be reliable?

Yes, they are reliable.Type your answers here.

- Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values.

The SRC Port is 1027, the DEST PORT is 80, the SEQUENCE NUM is 0 and the ACK NUM is 0.Type your answers here.

Look at the value in the Flags field, which is located next to the Window field. The values to the right of the “b” represent the TCP flags that are set for this stage of the data conversation. Each of the six places corresponds to a flag. The presence of a “1” in any place indicates that the flag is set. More than one flag can be set at a time. The values for the flags are shown below.

| Flag Place | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Value | URG | ACK | PSH | RST | SYN | FIN |

#### Question:

Which TCP flags are set in this PDU?

None of the flags are set.Type your answers here.

Close the PDU and click Capture/Forward until a PDU with a checkmark returns to the HTTP Client.

Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

The port numbers are the same and the sequence and ack numbers are both 1 now.Type your answers here.

Click the HTTP PDU which HTTP Client has prepared to send to MultiServer. This is the beginning of the HTTP communication. Click this second PDU envelope and select Outbound PDU Details.

#### Question:

What information is now listed in the TCP section? How are the port and sequence numbers different from the previous two PDUs?

It is the same as the last pdu.Type your answers here.

Reset the simulation.

### Examine FTP traffic as the clients communicate with the server.

Open the command prompt on the FTP Client desktop. Initiate an FTP connection by entering ftp 192.168.1.254.

In the Simulation Panel, change Edit Filters to display only FTP and TCP.

Click Capture/Forward. Click the second PDU envelope to open it.

Click the Outbound PDU Details tab and scroll down to the TCP section.

#### Question:

Are these communications considered to be reliable?

Yes, these communications are reliable. Type your answers here.

Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values.

#### Question:

What is the value in the flag field?

The flag value is 0b00000010.Type your answers here.

Close the PDU and click Capture/Forward until a PDU returns to the FTP Client with a checkmark.

Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

They are reversed, and the acknowledge number is now 1.ype your answers here.

Click the Outbound PDU Details tab.

#### Question:

How are the port and sequence numbers different from the previous results?

The numbers are revered and the ack number is 1.Type your answers here.

Close the PDU and click Capture/Forward until a second PDU returns to the FTP Client. The PDU is a different color.

Open the PDU and select Inbound PDU Details. Scroll down past the TCP section.

#### Question:

What is the message from the server?

Welcome to PT ftp server.Type your answers here.

Click Reset Simulation.

### Examine DNS traffic as the clients communicate with the server.

Repeat the steps in Part 1 to create DNS traffic.

In the Simulation Panel, change Edit Filters to display only DNS and UDP.

Click the PDU envelope to open it.

Look at the OSI Model details for the outbound PDU.

#### Question:

What is the Layer 4 protocol?

The protocol is UDP.Type your answers here.

Are these communications considered to be reliable?

No they are not reliable.Type your answers here.

Open the Outbound PDU Details tab and find the UDP section of the PDU formats. Record the SRC PORT and DEST PORT values.

#### Question:

Why are there no sequence and acknowledgement numbers?

SRC 1025, DEST 53. UDP doesn’t use a sequence to connect to servers. Type your answers here.

Close the PDU and click Capture/Forward until a PDU with a check mark returns to the DNS Client.

Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

They are reversed. your answers here.

What is the last section of the PDU called? What is the IP address for the name multiserver.pt.ptu?

DNS answer, 192.168.1.254.Type your answers here.

Click Reset Simulation.

### Examine email traffic as the clients communicate with the server.

Repeat the steps in Part 1 to send an email to user@multiserver.pt.ptu.

In the Simulation Panel, change Edit Filters to display only POP3, SMTP and TCP.

Click the first PDU envelope to open it.

Click the Outbound PDU Details tab and scroll down to the last section.

#### Questions:

What transport layer protocol does email traffic use?

It uses TCP.Type your answers here.

Are these communications considered to be reliable?

Yes they are reliable.Type your answers here.

Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values. What is the flag field value? DEST 1025, SRC 25, SEQ 0, ACK 0, 0b00000010.

Close the PDU and click Capture/Forward until a PDU returns to the E-Mail Client with a checkmark.

Click the TCP PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

They are reversed. The ACK number is now 1.Type your answers here.

Click the Outbound PDU Details tab.

#### Question:

How are the port and sequence numbers different from the previous two results?

The port numbers are reversed, the ACK and SEQ numbers are both 1.Type your answers here.

There is a second PDU of a different color that E-Mail Client has prepared to send to MultiServer. This is the beginning of the email communication. Click this second PDU envelope and select Outbound PDU Details.

#### Questions:

How are the port and sequence numbers different from the previous two PDUs?

The port numbers are reversed, the ACK and SEQ numbers are both 1.Type your answers here.

What email protocol is associated with TCP port 25? What protocol is associated with TCP port 110?

SMTP and POP3. your answers here.

End of document