Net 1061 Switching, Routing, and Wireless Essentials

Name:

Lab:  #

Follow the instructions down below for the lab itself. For this lab, all answers need to be in blue font. For the questions right below, answer in complete sentences. If this is a self-grading packet tracer. Ensure you paste the screen shot of your score page at the bottom of the document. You will upload both this document and the pkt file regardless if it is self-grading or not. Let the instructor know if you have any questions.

Lab Analysis Report

1.  Using complete sentences summarize work you completed during the lab.

In this lab, I troubleshooted the routing problems in this network. First, I checked the IPv4 static routes on all the routers. Then, I checked the IPv4 default routes on the routers. Then, I checked the IPv6 routes on all the routers. Finally, I verified connectivity between the devices on the network.

2. Using complete sentences describe what you learned from the lab. Hint; look at the lab objectives listed at the top of the lab section.

In this lab, I learned how to take my skills of knowing how to make IP routes, and applied them in a real-world scenario where connectivity needed to be troubleshooted.

Problems Encountered

1.  Using complete sentences describe any problem(s) experienced during lab.

I didn’t experience any problems.

2. Using complete sentences describe how you solved your problem(s).

3. Using complete sentences explain if you needed any assistance with the lab; then list what you learned from that assistance.

# Packet Tracer - Troubleshoot Static and Default Routes

# Addressing Table

| Device | Interface | IP Addresses |
| --- | --- | --- |
| R1 | G0/0 | 172.31.1.1/25 |
| R1 | G0/0 | 2001:DB8:1::1/64 |
| R1 | S0/0/0 | 172.31.1.194/30 |
| R1 | S0/0/0 | 2001:DB8:2::194/64 |
| R2 | G0/0 | 172.31.0.1/24 |
| R2 | G0/0 | 2001:DB8:3::1/64 |
| R2 | S0/0/0 | 172.31.1.193/30 |
| R2 | S0/0/0 | 2001:DB8:2::193/64 |
| R2 | S0/0/1 | 172.31.1.197/30 |
| R2 | S0/0/1 | 2001:DB8:4::197/64 |
| R3 | G0/0 | 172.31.1.129/26 |
| R3 | G0/0 | 2001:DB8:5::1/64 |
| R3 | S0/0/1 | 172.31.1.198/30 |
| R3 | S0/0/1 | 2001:DB8:4::198/64 |
| PC1 | NIC | 172.31.1.126/25 |
| PC1 | NIC | 2001:DB8:1::126/64 |
| PC2 | NIC | 172.31.0.254/24 |
| PC2 | NIC | 2001:DB8:3::254/64 |
| Server | NIC | 172.31.1.190/26 |
| Server | NIC | 2001:DB8:5::190/64 |

# Objectives

In this activity you will troubleshoot static and default routes and repair any errors that you find.

Troubleshoot IPv4 static routes.

Troubleshoot IPv4 default routes.

Troubleshoot IPv6 static routes.

Configure IPv4 static routes.

Configure IPv4 default routes.

Configure IPv6 static routes.

# Background / Scenario

A newly hired network technician is attempting to preconfigure a simple topology that will be delivered to a customer. The technician has not been able to establish connectivity between the three LANs. You have been asked to troubleshoot the topology and verify connectivity between the hosts on the three LANs over IPv4 and IPv6.

# Instructions

## Locate and document the problems.

Record your findings in a table like the one below.

| Location | Problem | Solution |
| --- | --- | --- |
| R2 | Static route to R1 was via R3’s interface | hangeblankChange the interface to R1’s |
| R2 | Static route to R3 was via R1’s interface | blankChange the interface to R3’shange |
| R2 | IPv6 route to R1 was typed in wrong | Address was corrected |
| R3 | No ipv6 route to R1 | IPv6 route was addedblank |
| R3 | IP route to R1 had the wrong maskblank | Correct mask was added |

## Repair the problems.

Configure the devices so that full connectivity exists between the hosts on the LANs over IPv4 and IPv6.

Note: Your task is to establish connectivity using the existing static route design. Changing the types of static routes used will result in a loss of points.

End of document